Superior University Faculty of CS & IT

Project Report

Computer Networks

Submitted by: Iqra Khawar

Submitted to: Sir Rasikh Ali

Section: 5B

Semester: 5th

Submission Deadline: 10-12-24

Project: Superior University Network GUI design and implementation

Roll no: su92-bssem-f22-307

Department of Software Engineering

Table of Contents

Project overview	3

Building layout of superior university	3

Connection Devices	3

Switches:	3

Router:	3

Server:	4

PC’s:	4

Design	4

Implementation of Network	4

Dynamic routing (DHCP)	4

Implementation of dynamic routing for each servers	5

Command	5

Routing Internet Protocol	7

Commands	7

VLAN	8

Inter-VLAN	8

Commands	9

DNS (Domain Name System)	14

Figure

Figure 1	4

Figure 2	5

Figure 3	7

Figure 4	11

Figure 5	11

Figure 6	12

# Project overview

My project title is “Superior University, Gold Campus”. Designing and implementation on cisco packet tracer of that platform. Implemented VLAN, Inter-VLAN, dynamic routing (DHCP) & RIP (Routing Internet Protocol), DNS (Domain Name System) and validation of message passing from one network to another network and from one VLAN to another VLAN.

## Building layout of superior university

Three Floors:

- Ground Floor:

Likely includes an IT room and some labs/offices.

- First and Second Floors:

Contain the remaining labs and administrative offices.

Rooms:

- 10 Labs:

For students' practical sessions, requiring robust connectivity and separate VLANs for efficient management.

- 3 Admin Offices:

These require secure connections for administrative tasks, possibly on a different VLAN.

- 1 IT Room:

The central point for managing the network, housing core switches, routers, and servers.

# Connection Devices

## Switches:

Each Labs has only one switch

## Router:

Only one router used in the platform.

## Server:

Only one server is used in the platform.

## PC’s:

Each Lab has 32 Pc’s. Admin Office has three PC’s. Total PC’s are 325.

# Design

# Implementation of Network

## Dynamic routing (DHCP)

I’ve have implemented dynamic routing and DHCP in your network design, where each network has a dedicated server, and dynamic routing connects all the servers. This approach is efficient and modular, ensuring scalability and seamless communication between networks.

Design Approach:

Servers:

Placed strategically in each network, handling local traffic and serving as gateways.

Dynamic Routing:

Used to interconnect these servers and enable seamless communication across networks.

## Implementation of dynamic routing for each servers

Figure 1

- Assign IP address to each server.

- Assign gateway to each server.

- Assign gateway in DHCP then save and on DHCP.

- Checking all the PCS IP addresses by clicking on DHCP.

### Command

DHCP POOL LAB1

ip dhcp pool Lab1

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

ip dhcp excluded-address 192.168.10.1 192.168.10.10

DHCP POOL LAB2

ip dhcp pool Lab2

network 192.168.12.0 255.255.255.0

default-router 192.168.12.1

ip dhcp excluded-address 192.168.12.1 192.168.12.10

DHCP POOL LAB3

ip dhcp pool Lab3

network 192.168.14.0 255.255.255.0

default-router 192.168.14.1

ip dhcp excluded-address 192.168.14.1 192.168.14.10

DHCP POOL LAB4

ip dhcp pool Lab4

network 192.168.11.0 255.255.255.0

default-router 192.168.11.1

ip dhcp excluded-address 192.168.11.1 192.168.11.10

DHCP POOL LAB5

ip dhcp pool Lab5

network 192.168.13.0 255.255.255.0

default-router 192.168.13.1

ip dhcp excluded-address 192.168.13.1 192.168.13.10

DHCP POOL LAB6

ip dhcp pool Lab6

network 192.168.18.0 255.255.255.0

default-router 192.168.18.1

ip dhcp excluded-address 192.168.18.1 192.168.18.10

DHCP POOL LAB7

ip dhcp pool Lab7

network 192.168.15.0 255.255.255.0

default-router 192.168.19.1

ip dhcp excluded-address 192.168.19.1 192.168.19.10

DHCP POOL LAB8

ip dhcp pool Lab8

network 192.168.21.0 255.255.255.0

default-router 192.168.21.1

ip dhcp excluded-address 192.168.21.1 192.168.21.10

DHCP POOL LAB9

ip dhcp pool Lab9

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

ip dhcp excluded-address 192.168.20.1 192.168.20.10

## Routing Internet Protocol

Ease of Configuration: RIP is straightforward to set up and manage.

Dynamic Updates: It automatically updates routing tables using periodic broadcasts, reducing manual intervention.

Every router connected to the dedicated servers in your network is configured to use RIP.

RIP ensures that routers exchange routing tables periodically, sharing information about reachable networks.

### Commands

(router-config)#Router rip

(router-config)#network <network address>

Figure 2

The purpose of routing is to pass data from one network to another network.

## VLAN

VLAN stands for Virtual Local Area Network.

VLAN Implementation:

- Three Networks with VLANs:

Each of the three networks is logically segmented into different VLANs.

VLANs ensure that devices within each network can communicate locally, but are isolated from devices in other VLANs unless explicitly allowed.

- Purpose of VLANs:

Security: Traffic is isolated, reducing the risk of unauthorized access.

Broadcast Control: Limits broadcast traffic to only devices within the same VLAN, improving efficiency.

Logical Segmentation: Allows grouping of devices (labs, admin, IT) regardless of their physical location.

## Inter-VLAN

Inter-VLAN Routing:

- Role of Inter-VLAN Routing:

Allows communication between VLANs (e.g., for sharing resources like servers or printers across networks).

This is typically configured on a Layer 3 switch or a router.

- Implementation:

Router-on-a-Stick: If using a router, a single physical interface is configured with subinterfaces, each representing a VLAN.

Layer 3 Switch: Directly performs routing between VLANs, eliminating the need for a router.

VLAN 30

Figure 3

VLAN 10

VLAN 20

### Commands

Applying VLAN 10, access & trunk ports

Switch>enable

Switch#config t

Switch(config-t)#vlan 10

Switch(config-t-vlan)#exit

Switch(config-t)#int fa0/3

Switch(config-t)#switchport access vlan 10

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/1

Switch(config-t)#switchport access vlan 10

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/5

Switch(config-t)#switchport access vlan 10

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/4

Switch(config-t)#switchport access vlan 10

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/8

Switch(config-t)#switchport access vlan 10

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int gig0/1

Switch(config-t)#no shutdown

Switch(config-t)#switchport access trunk

Switch(config-t)#exit

Applying encapsulation on router (vlan 10)

router(config-t)#int gig0/1.10

router(config-t)#encapsulation dot1Q vlan 10

router(config-t)#ip address 192.168.15.1 255.255.255.0

router(config-t)#exit

Applying VLAN 20, access & trunk ports

Switch>enable

Switch#config t

Switch(config-t)#vlan 20

Switch(config-t-vlan)#exit

Switch(config-t)#int fa0/1

Switch(config-t)#switchport access vlan 20

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/3

Switch(config-t)#switchport access vlan 20

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/5

Switch(config-t)#switchport access vlan 20

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/6

Switch(config-t)#switchport access vlan 20

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int gig0/1

Switch(config-t)#no shutdown

Switch(config-t)#switchport access trunk

Switch(config-t)#exit

Applying encapsulation on router (vlan 10)

router(config-t)#int gig0/1.20

router(config-t)#encapsulation dot1Q vlan 20

router(config-t)#ip address 192.168.17.1 255.255.255.0

router(config-t)#exit

Applying VLAN 30, access & trunk ports

Switch>enable

Switch#config t

Switch(config-t)#vlan 30

Switch(config-t-vlan)#exit

Switch(config-t)#int fa0/2

Switch(config-t)#switchport access vlan 30

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/3

Switch(config-t)#switchport access vlan 30

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/1

Switch(config-t)#switchport access vlan 30

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/4

Switch(config-t)#switchport access vlan 30

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int fa0/5

Switch(config-t)#switchport access vlan 30

Switch(config-t)#switchport access mode

Switch(config-t)#exit

Switch(config-t)#int gig0/1

Switch(config-t)#no shutdown

Switch(config-t)#switchport access trunk

Switch(config-t)#exit

Applying encapsulation on router (vlan 30)

router(config-t)#int gig0/1.30

router(config-t)#encapsulation dot1Q vlan 30

router(config-t)#ip address 192.168.16.1 255.255.255.0

router(config-t)#exit

Figure 4

Figure 5

Figure 6

## DNS (Domain Name System)

DNS Implementation in Your Server:

- Purpose of DNS:

Translates domain names (e.g., lab1.superior.edu) into IP addresses (e.g., 192.168.10.5) and vice versa.

Makes it easier for users to access resources without remembering IP addresses.

- DNS Server Role:

Acts as the central point for resolving names in your network.

Can be configured to handle both internal domain names (e.g., lab networks, admin systems) and external domain names (e.g., internet access).

- Integration with DHCP:

DHCP server assigns IP addresses to devices, and the DNS server resolves the corresponding names.

The DHCP server can be configured to automatically update the DNS server with assigned hostnames and IPs.

DNS applied on all servers.